# SmartBranch 360

Secure Branch Office Network — Complete Project Report

Cisco AICTE Virtual Internship Program 2026 — Project 1: Networking + Packet Tracer + Python

Student: Sidharth Kumar

# 1. Objective

Design and build a small branch-office network in Cisco Packet Tracer supporting wired employees, guest Wi-Fi, an internal server, internet access via NAT, and secure out-of-band management — then validate the design with a Python assurance tool and prove resilience through five fault-injection/troubleshooting cycles, all with verified evidence.

# 2. Topology Overview

The branch uses one ISR router (R1) performing router-on-a-stick inter-VLAN routing and NAT, a two-tier switch layer (SW1 distribution, SW2 access), one wireless access point (AP1) serving the guest SSID, one internal server (SRV1), and 9 endpoints (6 wired employee PCs, 1 wired management PC, 2 wireless guest devices).

- R1 — GigabitEthernet0/0 trunked to SW1 (sub-interfaces per VLAN); GigabitEthernet0/1 faces the Internet cloud, NAT-outside.

- SW1 — Distribution switch; trunks to R1, SW2, AP1; SRV1 and Management PC on access ports.

- SW2 — Access switch; 6 wired employee PCs on access ports; trunk uplink to SW1.

- AP1 — Broadcasts guest SSID 'SmartBranch-Guest' (WPA2-PSK/AES); wired uplink is an access port in VLAN 20.

Built topology in Packet Tracer: R1, SW1, SW2, AP1, SRV1, and all 9 endpoints connected.

All links verified up (green) after configuration — every port active and forwarding correctly.

# 3. VLAN and IP Address Plan

| VLAN | Name | Subnet | Gateway | Address Method |
| --- | --- | --- | --- | --- |
| 10 | EMPLOYEE | 10.10.10.0/24 | 10.10.10.1 | DHCP (.100–.200) |
| 20 | GUEST | 10.10.20.0/24 | 10.10.20.1 | DHCP (.50–.150), wireless |
| 30 | SERVER | 10.10.30.0/24 | 10.10.30.1 | Static (SRV1 = .10) |
| 99 | MANAGEMENT | 10.10.99.0/24 | 10.10.99.1 | Static (MgmtPC = .2) |

# 4. Services Configured

- DHCP: per-VLAN pools on R1 for EMPLOYEE and GUEST; SERVER and MANAGEMENT use static addressing.

- DNS: forwarded to 8.8.8.8 by default; an internal DNS record was also added on SRV1 during Fault 4 troubleshooting (see Section 6).

- NAT: PAT (overload) on R1's outside interface (Gi0/1) for EMPLOYEE and GUEST subnets.

- Inter-VLAN routing: router-on-a-stick using dot1Q sub-interfaces Gi0/0.10 / .20 / .30 / .99 on R1.

- Wireless: AP1 broadcasts SSID 'SmartBranch-Guest' with WPA2-PSK/AES authentication, mapped to VLAN 20.

AP1 wireless configuration — SSID and WPA2-PSK/AES authentication set.

# 5. Security Rules

| Rule | Mechanism | Effect |
| --- | --- | --- |
| Guest isolation | Extended ACL GUEST_ISOLATION on R1 (Gi0/0.20 inbound) | Denies GUEST -> SERVER and GUEST -> MANAGEMENT; permits internet traffic. |
| NAT scope | Standard ACL NAT_ACL + 'ip nat inside source list ... overload' | Only EMPLOYEE and GUEST subnets are translated to the internet; SERVER/MANAGEMENT are not exposed. |
| Wireless security | WPA2-PSK/AES on AP1 | Guest SSID requires authentication, preventing open/unauthenticated wireless access. |

# 6. Verification Testing

Connectivity and isolation were verified directly via ping between hosts on different VLANs.

| Test | Result |
| --- | --- |
| Employee (PC1/PC2) -> Server (10.10.30.10) | Success — replies received |
| Employee -> Internet (8.8.8.8) | Success — replies received |
| Guest (PC7/PC8, wireless) -> Server | Blocked — 'Destination host unreachable' (ACL working as intended) |
| Guest -> Management (10.10.99.x) | Blocked — 'Destination host unreachable' |
| Guest -> Internet | Success — replies received (guest is isolated from internal VLANs only, not the internet) |

PC7 connected wirelessly to the guest SSID, received 10.10.20.50 via DHCP.

PC8 connected wirelessly to the guest SSID, received 10.10.20.51 via DHCP.

Internet reachability confirmed end-to-end (ping 8.8.8.8, 0% loss) after NAT configuration.

R1 CLI — GUEST_ISOLATION ACL and NAT sub-interface (ip nat inside) configuration on R1.

# 7. Fault Injection & Troubleshooting (5 Required Faults)

Each fault was deliberately injected, diagnosed from show-command / host evidence, fixed, and re-verified. Full before-and-after evidence follows.

## Fault 1 — Wrong Default Gateway (Layer 3)

Injection: PC2 manually set with an incorrect gateway (10.10.10.254 instead of 10.10.10.1).

PC2 IP configuration showing the incorrect gateway 10.10.10.254 (injected fault).

Symptom: pings from PC2 to the server and to the internet both failed.

PC2 — ping 10.10.30.10 and ping 8.8.8.8 both time out with the wrong gateway set.

ipconfig confirms the faulty gateway (10.10.10.254) on PC2's FastEthernet0 interface.

Fix: gateway corrected back to 10.10.10.1. Re-test confirmed connectivity restored.

PC2 — after correcting the gateway, ping 10.10.30.10 and ping 8.8.8.8 both succeed (0% loss).

Root cause: incorrect default gateway prevents any off-subnet routing — a Layer 3 addressing fault.

## Fault 2 — Missing VLAN on Trunk (Layer 2)

Injection: VLAN 20 (GUEST) removed from the SW1<->R1 and SW2<->SW1 trunk allowed-list on both switches.

SW1 — 'switchport trunk allowed vlan remove 20' applied; VLAN 20 no longer in the trunk allowed list.

SW2 — same fault applied; trunk allowed list now only shows 10,99.

Symptom: guest device PC7 could not renew its DHCP lease at all.

PC7 — DHCP release/renew fails ('DHCP request failed') because guest broadcast traffic has no path to R1.

Fix: VLAN 20 re-added to both trunks.

SW1 — VLAN 20 restored; show interfaces trunk now lists 10,20,30,99 on both Fa0/1 and Fa0/2.

SW2 — VLAN 20 restored on the Fa0/1 trunk to SW1.

PC7 — DHCP renew now succeeds: 10.10.20.52 assigned, correct gateway and DNS.

Root cause: VLAN 20 missing from the trunk allowed-list blocked all guest traffic — a Layer 2 trunking fault.

## Fault 3 — DHCP Pool Exhausted (Layer 3 / DHCP)

Injection: R1's excluded-address range for the EMPLOYEE pool changed to 10.10.10.1-10.10.10.254, leaving almost no addresses to lease.

R1 — excluded-address range widened to 10.10.10.1-10.10.10.254 (injected fault).

Symptom: PC3, an existing employee host, failed to renew its DHCP lease.

PC3 — DHCP renew fails ('DHCP request failed') with the pool exhausted.

Fix: excluded-address range corrected back to 10.10.10.1-10.10.10.99.

R1 — corrected excluded-address range confirmed via show run | section dhcp.

PC3 — DHCP renew now succeeds: 10.10.10.102 assigned with correct gateway.

Root cause: an overly broad excluded-address range starves the DHCP pool — a configuration fault, not a link/hardware fault.

## Fault 4 — ACL Blocking DNS (Layer 4/7)

Injection: the GUEST_ISOLATION ACL contained a broad 'deny udp any any eq 53' rule, blocking all DNS resolution for the guest VLAN even though IP-based reachability still worked.

nslookup google.com times out repeatedly before finally resolving — intermittent/blocked DNS behaviour.

Fix: the broad deny was removed; an explicit permit for UDP/53 to an internal DNS host was added ahead of the guest-isolation deny rules, and SRV1 was configured as an internal DNS server (google.com -> 10.10.30.10) for verification.

SRV1 — DNS service enabled with an A record for google.com pointing to 10.10.30.10.

R1 — GUEST_ISOLATION ACL updated: broad DNS deny removed, explicit permit for UDP/53 added; show access-lists confirms the corrected rule order and match counts.

Root cause: an ACL rule blocking UDP/53 breaks name resolution independent of underlying IP connectivity — an Application/Transport-layer fault.

## Fault 5 — NAT ACL Missing Guest Subnet (Layer 3 / NAT)

Injection: the GUEST subnet (10.10.20.0/24) removed from NAT_ACL, so R1 no longer translated guest traffic for internet access.

R1 — NAT_ACL no longer permits 10.10.20.0/24; show ip nat translations shows zero active translations.

Symptom: guest devices lost internet access entirely while the employee VLAN was unaffected.

Fix: guest subnet re-added to NAT_ACL.

R1 — NAT_ACL corrected; show access-lists confirms both EMPLOYEE and GUEST subnets are now permitted for NAT.

PC7 (guest) — ping 8.8.8.8 now succeeds with 4/4 replies (0% loss), confirming internet access restored.

Root cause: NAT only translates traffic matched by NAT_ACL — omitting a subnet silently blocks its internet access even though routing is otherwise fine.

# 8. Python Assurance Tool

network_checker.py performs two kinds of validation: (1) static checks of the YAML VLAN/IP plan — duplicate VLAN IDs, overlapping subnets, gateway-in-subnet correctness, guest-isolation policy — and (2) dynamic checks of pasted show-command output — missing VLANs on trunks, interfaces down, duplicate-IP log entries, empty NAT translation tables. It was run against real show-command output captured from the built topology:

network_checker.py run in a real terminal against captured show-command output from R1, SW1, and SW2. Plan check passed cleanly; two unused interfaces (Gi0/2, Vlan1) were correctly flagged as administratively down — expected, since they are not part of this design (VLAN 99 is used for management instead of the default VLAN1).

# 9. Summary

| Fault | OSI Layer | Status |
| --- | --- | --- |
| 1. Wrong default gateway | Layer 3 | Diagnosed, fixed, re-verified |
| 2. Missing VLAN on trunk | Layer 2 | Diagnosed, fixed, re-verified |
| 3. DHCP pool exhausted | Layer 3 / DHCP | Diagnosed, fixed, re-verified |
| 4. ACL blocking DNS | Layer 4/7 | Diagnosed, fixed, re-verified |
| 5. NAT ACL missing subnet | Layer 3 / NAT | Diagnosed, fixed, re-verified |

All required components are complete: working topology with VLAN segmentation, DHCP, NAT, guest isolation and wireless; 5 fault-injection/fix cycles with full evidence; and an independently working Python assurance tool validated against real captured output. Remaining submission steps: save the Packet Tracer file as SmartBranch360.pkt and record the 5-10 minute demo video walking through this report.